SOLAR THERMAL AND PHOTOVOLTAIC POWER PLANTS

- - TECHNICAL SOLUTIONS AND PROSPECTS

FOR SOUTHERN CROATIA

Natko Urli, Claudia Jedriško*, Damir Spinčić*, and Branimir Hrastnik#

Rudjer Bošković Institute, Bijenička 54, 10000 Zagreb

*Faculty of Engineering University of Rijeka, Vukovarska 58, 51000 Rijeka

#Energy Institute “Hrvoje Požar”, Ulica grada Vukovara 37, 10000 Zagreb

Phone: ++385-1-4680-210; Fax: ++385-1-4680-114, E-mail: urli rudjer.irb.hr

Abstract: Only fossil fuel and to a lesser degree hydro-power plants are planned to meet an increased electricity demand in Croatia in the first decade of the next century. However, due to a relatively high solar potential in the southern part of Croatia, and because of extremely fast development of solar thermal power technologies as well as photovoltaics in the world in the last couple of years, it is quite probable that the price of solar electricity may become competitive sooner than expected. Therefore, it is necessary to initiate without delays demonstration and prototype projects both in solar thermal and photovoltaic fields in order to be prepared for introduction of these renewable energy technologies in electricity production capacities of the country, thus preserving clean environment along the Adriatic coast and making possible further sustainable development of tourism.

Key words: solar thermal, photovoltaics, power plants, technology development

- INTRODUCTION

Installed capacity of electric power plants in Croatia amounts to 3415 MW. It consists of 2076 MW in 17 hydro-electric and 1339 MW in oil- and gas-powered plants. Additionally, 332 MW are owned in the nuclear power plant Krsko in Slovenia, and 650 MW in the coal-fired plants in Serbia and Bosnia-Herzegovina. It is questionable, depending upon political situation, whether these fossil-fired plants in other republics of former Yugoslavia can be utilized in future.

In 1997, the total electricity production including the corresponding contribution of the nuclear power plant was equal to 11,231 kWh, covering about 90 percent of the total electricity demand. The remaining of 10 percent was the balance of import and export of electricity with neighboring countries. A relatively large proportion of international exchange was caused by disconnectedness between northern and southern part of the Croatian power system and by varying hydrological conditions during the year.

In a proposed “Strategy on Energy Development of Republic of Croatia” 1  it turned out that from 1300 to 1500 MW additional electricity generating capacity should be installed till 2010 in order to meet the total projected production of 21 - 23 TWh. Unfortunately, only high-cost hydro, gas-fired and imported coal-fired power plants have been envisaged in the planning. In the critical, southern part of the country, not enough exploitable hydro potential has been left, and there is no gas pipe-line system. Therefore, only coal-fired thermal power plants have been proposed with several locations along the Adriatic coast, resulting in strong opposition of tourism oriented local population and national organizations for protection of environment.

The geographical position of Croatia offers a large possibility of harnessing solar energy with an average daily global radiation of 3.6 kWh/m2 up to almost 2000 kWh/m2 on the optimally inclined surface at the most favorable locations in Dalmatia. Its technical potential is enormous reaching more than 700 TWh that is about 13 times the present total energy consumption in Croatia. Contrary to the situation in Western and Central Europe, it is not limited by lack of suitable space for solar installations.

Of course, only a small portion of this huge potential is presently economically competitive in comparison to other conventional energy sources, either due to higher cost or lack of suitable energy storage systems. However, because of recent extraordinary progress in development of some solar technologies and unacceptable global environmental effects by extensive fossil fuel utilization, we may expect a significant penetration of solar energy and some other new renewable sources in energy supply of many countries including Croatia. This should affect both thermal as well electric energy production and consumption.

2. DEVELOPMENT OF SOLAR THERMAL POWER PLANTS

Solar thermal power (STP) plants make use of steam to drive turbines, as in conventional power plants, but the steam is produced using heat of the sun. Production is emission-free. It is possible to combine this technology with conventional power to create hybrid systems.

There are three main technologies realized and tested in practice: a) STP with line-focusing collectors, b) STP with point-focusing collectors, and c) central receiver STP with heliostat fields. All concentrating collectors utilize only direct normal solar radiation, that is, they require 1- or 2-axes daily tracking of the sun. Recent studies on STP 2  predict that by the year 2025 it should be both technically and economically viable to create 23,000 MW of such units in total in the Mediterranean region.

2.1 STP with Line-Focusing Parabolic Trough

The parabolic trough collectors are set out in long rows (up to 100 meters in length), running north-south, and track the sun from east to west during each day. An absorber pipe is placed along the focal line, and the parabolic mirror concentrating the solar radiation onto the pipe (with the lowest concentrating ratio of 10 - 100 of all three concentrating systems). The sun’s heat is transferred to the fluid (generally a synthetic oil) flowing in the pipe. By heat exchangers it converts water into steam (superheated) to 390 oC which in turn drives the power-generating turbines. The heat produced can be used as well as process heat for industry.

This technology is the only one used worldwide for commercial electricity production since 1985 starting from 14 MW SEGS unit up to 80 MW LUZ power plants in California with a total area of about 2.5 million square meters of parabolic trough collectors supplying more than 4000 GWh into the electric grid. The unit cost of the SEGS plants dropped from over 4000 US$/KW to about 3000 US$/KW and the electricity cost to about 0.15 US$/kWh.

The most promising development in this technology enabling a further decrease in generating cost is the introduction of direct steam generation (DSG). Instead of oil, a mixture of water and steam is used in the absorber pipes as the heat transfer medium avoiding heat exchangers with intermediate transfer medium. Prototypes of such systems have been tested at the Plataforma Solar de Almeria in Spain. The European Communities initiated transfer and further development of parabolic trough power plants to the Mediterranean countries in order to decrease cost of solar electricity to 0.08 - 0.11 US$/kWh such as in THESEUS project in Crete, Greece. It consists of a nominal 52 MWe net solar power plant with an advanced parabolic trough collector field as the primary heat source. The power block is a conventional Rankine-cycle reheat steam turbine with its associated balance of plant equipment. The solar field energy source is supplemented with an oil-fired boiler to supply steam during conditions of low insolation. The solar field will consist of an advanced design based on the LS-3 parabolic troughs used at 80 MW SEGS-IX plant in California, but improved by a number of innovative features. THESEUS will be designed as an independent power producer (IPP) plant selling electricity to the Greek Public Power Corporation.

Similarly, a hybrid 137 MW solar/natural gas prototype power plant has been proposed for the location in Spain, and a 80 MW plant in Morocco with 3-hr thermal storage and supported by diesel fuel.

- STP with Point-Focusing Collectors (Dish Concentrators)

A dish/Stirling system operates by 2-axes tracking the sun and reflecting sunlight to the focus of the dish (paraboloidal mirror) where it is absorbed in a thermal receiver. The heat absorbed by the receiver is then transferred to the Stirling engine where it is converted to mechanical power turning electrical generator. The unit size of individual dish is defined by the dish diameter being typically up to 10 meter restricting the electric power output to about 25 KWe. Recently, Schlaich Bergmann und Partner of Stuttgart, Germany, and SAIC of San Diego/CA, USA, developed kinematic Stirling engine and a faceted, stretched-membrane dish with a peak solar-to-net electric efficiency of 21 %.

This technology (with the highest concentration ratios of 600 - 2000) is specially suited for smaller off-grid applications where it competes with conventional diesel systems.

- Central Receiver STP with Heliostat Fields

This technology offers concentration ratios from 300 - 1500 and temperatures of 550 - 1500 oC. The set-up is a field of heliostats mounted at ground level surrounding a central tower with a receiver. The heliostats reflect and concentrate the solar radiation onto the receiver where the heat is transferred to a heat transfer medium. Liquid as primary coolant and as a storage medium was employed in the form of eutectic salt in the 2.3 MWe Themis plant at Targasonne, France, and sodium in the IAE-SSPS 500 KWe plant in Almeria, Spain. IEA-SSPS plant was operated from late 1981 until August 1986 showing the technical feasibility and excellent component-level performance of a high-flux sodium receiver. However, high standards of quality control are required to avoid hazards and costly O&M associated with sodium equipment and heating elements for maintaining sodium above the freezing temperature.

Development of heliostats exhibited a trend from heavy construction with second glass mirrors to lightweight low-cost constructions with front-surface-silvered polymer foil reflectors or stressed membrane designs.

The largest 10 MWe SOLAR TWO central receiver power plant started to operate in California in 1996 with enough thermal storage to produce electricity for three hours after sunset at full capacity. It is intended to be a prototype of 100 - 200 MW hybrid solar/gas units at the end of this century with 15 % conversion efficiency, specific cost of 2000 US$/KWp, and the levelized cost of electricity of 6 -7 USc/kWh.

German PHOEBUS power plant project with central volumetric air receiver operating up to 700 oC, supported by a gas burner, and with large light-weight heliostats offers 30 MW units providing electricity at 0.10 - 0.15 US$/kWh.

- DEVELOPMENT OF PHOTOVOLTAIC (PV) MARKET AND TECHNOLOGY

Solar cells as quantum generators of electricity mutually connected in a solar module produce electricity in a most elegant way without moving parts, and as emission-free sources of energy. Exposed to sunlight they come to full power almost instantly, and produce electricity even during cloudy weather as they can utilize both diffuse as well direct component of solar radiation. However, there have been two major obstacles preventing their widespread implementation: a) high production cost, and b)  non-existence of a suitable economical storage for electricity converted from solar energy. The energy storage is prerequisite for most of practical applications for compensation of intermittence of the incoming solar radiation flux and for overcoming night and long-term low insolation periods.

In spite of all problems, the PV industry has experienced double-digit growth rate in the last four years supplying predominantly off-grid niche-market such as telecommunications, consumer systems for homes and campers, isolated villages, small lighting systems etc. However, a big step forward occurred with 126 MW of PV cells shipped in 1997. This was a substantial 42 percent increase from 1996 comparable only to recent rates of increase of wind energy utilization or much earlier in computer business. It was caused mainly by political and public support in Japan, and Germany and USA, and reaching now 800 MW of globally installed PV capacity. Single-crystal silicon has retained almost a 50 % market share due to production increase at Siemens Solar Industries, Sharp, BP Solar, and Solec International. Polycrystalline silicon increased to a 34 % of the market share in 1997 due to new production capacities of Kyocera, Photowatt, and Solarex, and amorphous silicon (a-Si) increased the yearly production from 11.7 MW to 15 MW.

Even more spectacular increase in a-Si PV cell production may be expected in 1998, as 25 MW of new a-Si plants came on line and began production: USSC´s 5 MW triple-stack cells in Troy, MI, Amoco/Enron Solarex´s 10 MW double junction cell plant in James City, VA, and Canon´s 10 MW triple junction a-Si plant in Japan.

- Regional Trends

Japanese government tax incentives and subsidies program provoked installation of 9,400 PV systems in Japanese homes in 1997 aiming to 70,000 PV-powered homes by the year 2000. Subsidies will phase-out by 2001 when the PV technology should reach fully economic status! Japanese NEDO program predicts 4.6 GW of PV-power to be installed in Japan by 2010.

Europe is not lagging behind Japan as it increased PV production by 56 % from 18.8 MW to 30.4 MW during 1997. This trend will continue by additional 40 MW production by 2000 when only Pilkington Solar International and Shell will account for 25 MW. This is in line with EU White Paper on Energy for the Future 3  asking for hundred-fold increase of PV from the present 30 MW level to 3 GW in 2010.

US companies Solarex, ASE Americas, USSC, Astropower and Siemens increased their shipments by 36 % from 39 MW to 51 MW. President Clinton´s Million Solar Roofs Initiative by 2010 will also give a strong support for further development of the American PV industry.

- Cost of Solar Modules

The present average price of solar PV modules on the global market is still high and equal to 4.2 US$/W preventing their wider implementation in countries with low-cost electricity from conventional sources of energy. There are little hopes that further development in single-crystal silicon technology may drop the cost below 2 US$/W. However,  three thin-film technologies, namely a-Si, CIGS, and CdTe have equal chances to become economical by 2005 reaching 0.60 ECU/W 4.

- Solar PV Power Plants

There are several experimental solar PV plants in the world, the largest one 3.3 MW at Serre, Italy 5,  in operation since 1995 gathering a valuable operating experience for much larger PV power plants in future.

In June 1997 Greek government approved an investment of 120 million of US$ for erection of 50 MW PV power plant on the island of Crete 6 to be put in operation by 2003.

In the meantime, a large market opened for solar PV modules: their integration into buildings as integral roof modules, roofing tiles and shingles, modules for vertical wall facades, sloped glassing systems and skylights.

- Long-Term Solar PV Electricity Prospects

In fall of 1997 Shell, the largest oil company in the world, entered the PV market following the example of another oil giant British Petroleum (BP). According to  their world energy scenario the world energy demand will triple during the next 50 years, and 50 % of this demand will be met by renewable energy sources in 2050. Photovoltaics will have 10 % share in these requirements starting from present negligible 0.004 % and reaching 0.13 % by 2010 (by maintaining 30 % annual growth rate!). 1 % is going to be attained in 2020, and from 5 to 10 % between 2030 and 2050.

- SOLAR ELECTRICITY IN SOUTHERN CROATIA

Taking into account significant results in solar electricity implementation around the world, Croatian national SUNEN program 7 has planned several demonstration and prototype projects such as: passive solar houses with integrated PV modules and thermal collectors, solar tourist settlements, and small autonomous PV, fuel cell, and reverse osmosis units for islands.

Projects requiring higher investments and leading to prototypes for further multiplication and commercial exploitation encompass: 100 kW hybrid PV solar-wind grid-connected power plant 8, 5 to 10 MW hybrid solar thermal-LNG cogeneration power plant, and 100 MW solar thermal power plant with a large thermal storage 9 .

- CONCLUSIONS

Although 100 kW PV solar power plant erected on the island of Vis, the sunniest region of Croatia, would provide electricity at still prohibitively high cost of about 0.20 US$/kWh, it will serve the purpose of long-term testing of domestically produced Končar a-Si modules of high resistance to degradation 10 as well as for getting experience in designing, constructing, and operating a PV power plant. Later, this could be very valuable in acceptance and transfer of technology of larger PV power plants in the MW range.

A hybrid solar thermal-LNG cogeneration power plant should serve as a prototype for at least 10 additional units to be installed on the Adriatic islands providing all local energy needs in heat and electricity, preventing degradation of environment, and preserving this region for further development of tourism.

The largest project, SOLENTEC 100 MW solar thermal power plant, presently in the final preparatory stage, may contribute significantly to fulfillment of the Croatian obligations taken at the 1997 Kyoto conference concerning the limits of emission of the so-called “greenhouse” gases to the atmosphere.

REFERENCES

1  Granić, G. et al.: Strategija energetskog razvitka Republike Hrvatske: nacrt, Zagreb,

Ministarstvo gospodarstva Republike Hrvatske: Energetski institut “Hrvoje Požar”, 1998.

2  Renewable Energy World, Jesch, L.: Solar thermal power, Vol.1, No.2 (1998) p.52.

3  White Paper for a Community Strategy and Action Plan, Energy for the Future:

Renewable Sources of Energy, Commision of the European Communities, Brussels,

26.11.1997, COM(97) 599 final.

4  Proc. 14th European PVSEC, Woodcock, J. et al.: A study of the upscaling of thin film

solar cell manufacture towards 500 MWp per annum, Barcelona, 1997.

5  World Renewable Energy Congress V, Vigoti, R. and Iliceto, A.: The largest PV

installation in Europe: perspectives of multimegawatt PV, ed. by A.A.M. Sayigh,

Pergamon Press, Part I, (1998) p.48.

6  Athens News, Crete power station beams in solar age, 13 June 1997.

7  SUNEN - Program korištenja sunčeve energije, Energetski institut “Hrvoje Požar”,

Zagreb, 1998.

8  World Renewable Energy Congress V, Urli, N.B. and Kamenski, M.: Hybrid photovoltaic/

wind grid-connected power plants in Croatian renewable eneegy program, ed. by A.A.M.

Sayigh, Pergamon Press, Part I, (1998) p.594.

9  Energy and the Environment,  Urli, N.: Solar thermal power plants - ecologically most

favorable electricity producers, Vol. I, Opatija (1996) p.117.

10 2nd World Conference and Exhibition on Photovoltaic Solar Energy Conversion,

Urli, N.B.: Influence of Technological Processes on the Initial and Stabilized

Performance of a-Si:H Large Area Modules, Vienna, 1998 (to be published).

SUNČANE I FOTONAPONSKE ELEKTRANE - TEHNIČKA RJEŠENJA I IZGLEDI ZA JUŽNU HRVATSKU

Sažetak: Samo fosilna goriva i u manjem obimu hidroelektrane planirane su za rješavanje rastuće potrebe za energijom u Hrvatskoj u prvome desetljeću sljedećeg stoljeća. S obzirom na relativno visoki potencijal suneve energije u južnom dijelu Hrvatske i zbog veoma brzoga razvoja tehnologija toplinske i fotonaponske porabe sunčeve energije u svijetu u posljednjih nekoliko godina, vrlo je vjerojatno da će cijena solarne električne energije postati kompetitivna skorije nego što se očekivalo. Stoga, potrebno je pravovremeno započeti izradu demonstracijskih pilot projekata, kako u području toplinske pretvorbe i fotonaponskih instalacija, da bi se pripremilo za uvođenje ovih obnovljivih izvora energije u kapacitete za proizvodnju električne energije u zemlji, čime bi se osigurao čisti okoliš duž Jadranske obale i omogućio daljnji održivi razvitak turizma.

Ključne riječi: toplinska uporaba sunčeve energije, fotonapon, elektrane, razvoj tehnologije