TOWARD LOW NOx HYDROGEN FUELED

INTERNAL COMBUSTION ENGINE

Vladimir MEDICA, Bernard FRANKOVIC

Faculty of Engineering

University of Rijeka

Rijeka, CROATIA

SUMMARY

Nitric oxides (NOx) are the main pollutants from hydrogen fueled internal combustion engine. With appropriate engine design and process management, NOx formation will be reduced to the source and environmental protection regulations will be met without expensive exhaust gas aftertreatment. The paper presents a numerical analysis of a hydrogen fueled spark ignition 4 cylinders inline engine. The analysis was performed using a phenomenological model for mixture preparation and combustion with detailed N-H-O chemistry. Lean burn concepts are analyzed, using mixture stratification. To increase the engine mean effective pressure, various supercharging levels have been analysed. The results of the analysis indicate the possibility of keeping the engine NOx emission levels under 3 g/kWh. In this sense the engine power droped up to 40% when compared with the same gasoline fueled engine.

1. INTRODUCTION

Hydrogen is the fuel for the future, not only as a conventional fuel for combustion, but as a fuel for nuclear fusion too. Hydrogen fuel powers the main engine of the orbiter in the US Space Shuttle program 1. Ideas in using hydrogen as fuel for internal combustion engines are about 60 years old 2. The hydrogen fueled spark ignition engines in the 1970s had about 30% lower power than the same engines fueled by gasoline. Many problems in early hydrogen fueled engines have been detected, as, for example, fires in inlet manifold and very fast combustion, like explosions, with high levels of NOx emission. These problems are caused mainly by premixed combustion mixture which is prepared before entering engine cylinders. A hydrogen fueled engine with premixed stoichiometric combustion mixture has high power with unacceptable NOx emission levels, without the possibility of lowering it under acceptable limits.

A hydrogen fueled internal combustion engine is competitive in efficiency, space requirement and price when compared with fuel cell. Nitric oxides are the main pollutant from hydrogen combustion in the atmosphere. Besides them, a very small amount of additional pollution originates from lubricating oil. At the present time of environmental technology, there is no cheap or simple device available for exhaust gas aftertreatment to reduce the NOx in the exhaust gases of internal combustion engines, except when using stoichiometric combustion mixtures with hydrocarbon fuels. This fact is valid for hydrogen fueled engine too. With appropriate engine design and process management, NOx formation may be reduced to the source and environmental regulations (Table 1) will be satisfied without exhaust gas aftertreatment. To achieve this goal, it is necessary to adopt the direct fuel injection, the lean burn concept, using charge stratification. Such combustion mixture preparation gives some advantages when compared with feeding the premixed combustion mixture to engine cylinders. There is no danger of inlet manifold fires. The combustion (or heat release) rate in the engine cylinders is mainly controlled by mixture preparation rate. Such combustion controlling helps to keep the NOx formation under control. Lower heat value of lean mixture can be compensated using supercharging and charge air cooling. Managing various strategies in mixture quality together with various supercharging levels, it is possible to reduce the NOx emission to acceptable levels at competitive fuel consumption and engine power. These strategies are analyzed in this paper.

2. ENGINE CONCEPT

A passenger car naturally aspirated spark ignition engine with 4 cylinders inline is used as the engine for numerical analysis. The original engine is contemporary gasoline fueled engine, with computerized engine process management control and catalytic exhaust gas aftertreatment. The main parameters of this engine are:

| Engine power | 60 kW |
| --- | --- |
| Nominal speed | 5000 rpm |
| Cylinder bore | 80.0 mm |
| Stroke | 69.5 mm |

The analysed engine is converted to use hydrogen as fuel. The engine is adapted to use inlet air supercharging and intercooling. The wastegate valve cuts the maximum supercharging pressure. The original compression ratio is increased from 11 to 13. Hydrogen is injected under pressure directly to the cylinders in the final stage of compression stroke. Injection is performed using central multiple hole injector. Injection pressure is regulated by control valve in a range between 30 and 100 bar. Injection pressure, duration and injection start have been varied in the analysis. Spark plugs are used to ignite the combustion mixture. Original engine speed is lowered to 4000 rpm to enable mixture formation by direct hydrogen injection. The engine scheme is presented in Fig. 1.

3. SIMULATION MODEL

The analysis is performed using a phenomenological model for mixture preparation and combustion with detailed H-N-O chemistry. Combustion space in this simulation model is divided into various homogenous zones. These zones comprise the hydrogen jet, fresh air and combustion products. When considering charge stratification, one more zone was added, to describe the stoichiometric mixture zone. The following assumptions are used in developing the simulation model:

- pressure is equalized through the combustion space,

- the fresh air zone and combustion products zone are of various compositions and temperatures, but they are homogenous through the same zone,

- the heat transfer between zones is possible only through enthalpy transport due to the mass transport,

- the blow-by from the combustion space is neglected and

- the working fluid is a mixture of ideal gases.

The simulation model is based on equations for mass, chemical species and energy conservation in each zone. In this paper, only a mathematical description of the high pressure part of the engine process is presented. The conservation equations are ordinary nonlinear differential equations. The derivation is performed on crank angle, rather than on the time basis:

( 1 )		      ddt=ddϕ dϕdt=ddϕ ω=2 π n ddϕ .

The mass conservation equation is:

( 2 )			dmidϕ=dmf,idϕ+dmffr,idϕ+dmj,idϕ ,

where the first term on the right side presents the fuel entering the i-th zone, the second term presents the transport from the flame front and the third one presents the transport from the j-th zone. A similar equation for s-th chemical species conservation is:

( 3 )		 dms,idϕ=dms,f,idϕ+dms,ffr,idϕ+dms,j,idϕ+kdms,k,idϕ ,

where first three terms present the same contribution as in equation (2), while the fourth term presents the species mass source from the k-th chemical reaction in the i-th zone.

For this case the energy conservation equation is:

( 4 )	       dUidϕ=hf,idmf,idϕ+hffr,idmffr,idϕ+hj,idmj,idϕ+kdQk,idϕ+dQEdϕ ,

where the last two terms on the right side present the heat contribution from the k-th chemical reaction and electric spark energy in the i-th zone.

The equation of state assumes the ideal gas properties of each species:

( 5 )			            pi Vi=Ti sms,i Rs .

The rate of k-th chemical reaction:

( 6 )		      νk,AA+νk,BB+νk,CC→νk,DD+νk,EE+νk,FF

is presented by equation:

( 7 )	dsk,idϕ≡dDkdϕ=νDω kf,k Aνk,ABνk,BCνk,C−kb,kDνk,DEνk,EFνk,F  .

The forward rate coefficients for the k-th chemical reaction are calculated from the Arrhenius equation:

( 8 )			         kf,k=AkTkbexp−EkRTk .

The backward rate coefficients are calculated using the following constant for chemical equilibrium:

( 9 )				    kb,k=Kp,kkf,k .

In the simulation model, the following set of 19 species have been considered: H2, H, O2, O, N2, N, OH, H2O, HO2, H2O2, HNO, HNO2, NH3, NH2, NH, N2H, NO, N2O and NO2. The set comprises 81 chemical reactions 5. The chemical species mass source and the energy source from chemical reactions in i-th zone are:

( 10 )       		       kdms,k,idϕ=Mskdsk,idϕ ,

( 11 )			     kdQk,idϕ=sMshskdsk,idϕ .

The start of the combustion was introduced by electric spark energy as spot source. The combustion rate of injected hydrogen is governed mainly by mixture preparation rate. The hydrogen injection is described as gaseous jet in the combustion space environment, taking also into consideration the charge flow. The combustion space geometry is considered using zone distribution in the combustion space. The mixture preparation rate and flame front supply with hydrogen and oxygen are modeled using gas jet theory with convective and diffusive mass transport.

The simulation model for complete engine system comprises also the equations for heat and mass transfer during engine fluid exchange process and complete system description (turbocharger, charge air cooler, wastegate valve, inlet and exhaust manifolds etc.) 6.

The set of nonlinear differential equations, describing engine process and engine system, is set up acording to the engine system composition. The solution of this equations is obtained using the Runge-Kutta-Fehlberg method with automatic integration step control.

4. RESULTS OF THE SIMULATIONS

In the simulations, various lean combustion mixtures are analysed for various mixture preparation parameters (injection begin, injection pressure) and ignition timing. To increase engine power, when operating with lean mixture, supercharging and air intercooling were applied.

Some results of numerical simulations are presented in Figures 2 to 5, where the NOx emission and specific fuel consumption are presented. Specific fuel consumption is recalculated to standard hydrocarbon fuel with lower heat value of 42.7 MJ/kg (while hydrogen has lower heat value of 120 MJ/kg).

Fig. 2 presents the influence of varying the hydrogen injection start on NOx emission and specific fuel consumption. The injection lasts until the injected fuel amount is sufficient to maintain the desired combustion mixture quality (lean mixture with air excess ratio  = 1.5). It is evident from Fig. 2 that the NOx emission decreases if injection start is closer to the top dead center (TDC) position. When the injection start is closer to the TDC, injection duration is longer due to higher cylinder pressures, so the mixture preparation and combustion rates are slower. The consequence of this decreased rates is higher specific fuel consumption. As the combustion mixture is not too lean, the NOx emissions are over 4 g/kWh.

Fig. 3 presents the influence of variation of the injection pressure on NOx emission and specific fuel consumption at various charging air pressures. At higher charging air pressure, the NOx emission is lower, mainly due the slower injection rate at increased cylinder pressures. This is also evident from the change in specific fuel consumption, while the maximum pressure and maximum temperature in the cylinder decrease due to the slower combustion rate. In the same time the energy losses from engine process are increased.

Fig. 4 presents the influence of combustion mixture quality, indicated by air excess ratio. Higher air excess ratio means leaner combustion mixture. To maintain the desired mean effective pressure, the supercharging air pressure was increased at leaner mixtures. Increasing the charging air pressure, higher mixture mass amount is inducted in engine cylinder to have the same fuel quantity for leaner mixture. Higher mass amount of combustion mixture, which is heated during combustion, lowers the end temperature when maintaining the same amount of heat added by combustion. As the consequence of lower temperatures, NOx emission was lowered under 1 g/kWh for air excess ratio of 2.76 for indicated parameters. The specific fuel consumption was practically equal. This fact presents very high potential to reduce the NOx emissions while maintaining engine efficiency at acceptable levels. For mixtures with air excess ratio higher than 1.8 the charge stratification was applied to ensure mixture ignition.

Fig. 5 presents the influence of ignition timing on NOx emission It presents common facts, which are already experienced in diesel engines. Delaying the fuel injection the NOx emissions decrease and specific fuel consumption increases. Very high penalties imposed by future emission regulations would lead to concepts with delayed ignition.

In all cases the engine power is lower than the original gasoline fueled spark ignition engine. This is due to the decreased nominal speed from 5000 to 4000 rpm to allow the mixture preparation by direct hydrogen injection and due to the limited possibilities of contemporary small scale turbochargers. In the worst case, hydrogen fueled engine had the power of 38 kW, which is 63% of original engine power.

5. CONCLUSION

The simulation model has indicated that hydrogen fueled direct injection engine has great potential to reduce the NOx emission is in order to meet future pollutant emission restrictions (presented in Table 1). Some possibilities of reducing NOx emission in the engine are only indicated from the results presented, to avoid the exhaust gas aftertreatment. It is evident that there exists the possibility of decreasing the NOx emission from hydrogen fueled direct injection engine under the limit of 3 g/kWh as proposed for the year 2005., while maintaining acceptable specific fuel consumption. This is achieved by proper matching of air supercharging level, hydrogen injection pressure and timing, ignition timing etc. Using the developed simulation model various strategies for computerized engine management control can be derived to meet future emission regulations.

LITERATURE

- - The World Book Encyclopedia, World Book, Inc., Chicago 1994., Vol. 9 (H), p.467

- Westerkamp L.: Der Erren-Wasserstoffmotor, ATZ 41 (1939), No. 19, p. 523-524

- Havenith  C., Meretei T., Romvari F.: Development of a RABA CNG Lean Burn Engine for Low Polluting City Bus Operation, Proceedings of the 18.th International Motor Symposium, Vienna, April, the 24-25, 1997.

- Hohlbaum B.: Beitrag zur rechnerischen Untersuchung der Stickstoffoxid-Bildung schnellaufenden Hochleistungsdieselmotoren, Dissertation, TU Hannover, 1992.

- Warnatz J., Maas U., Dibble R.W.: Combustion, Springer Verlag, Berlin, 1996.

- Medica V.: Simulation of turbocharged diesel engine driving electric generator under dynamic working conditions, Ph.D. Thesis, Technical faculty, University of Rijeka, 1988.

LIST OF SYMBOLS

| A...F A be h kf, kb Kp m M n p Q | chemical species designation molar concentration, mole/m3 specific fuel consumption, kg/kWh specific enthalpy, J/kg forward & backward rate coeff. chemical equilibrium constant mass, kg molar mass, kg/mole engine speed, s-1 pressure, Pa heat, J | R R t T U V     | gas constant, J/(kg.K) universal gas constant, J/(mol.K) time, s temperature, K internal energy, J volume, m3 crank angle, rad air excess ratio stoichiometric coefficient angular speed, s-1 |
| --- | --- | --- | --- |

Index list

| i j f ffr | current zone index neighboring zone index fuel flame front | E k s | electric spark chemical reaction index chemical species index |
| --- | --- | --- | --- |

Table 1 European limits for internal combustion engine derived pollutants (g/kWh) /3/

|  | EURO 2 | EURO 3 | EURO 4 | EURO 5 |
| --- | --- | --- | --- | --- |
| Year | 1996. | 2000 (discussion) | 2005 (discussion) | 2010 (discussion) |
| NOx Particulates HC CO | 7.0 0.15 1.1 4.0 | 5.0 0.10 0.7 2.5 | 3.0 0.08 0.5 1.0 | 1.5 0.05 0.3 1.0 |

Wastegate valve

Turbocharger

Hydrogen supply

Pressure reduction valve 300/(30...100) bar

Intercooler

P control

Fig. 1 Scheme of the analysed hydrogen fueled engine

be

be

g/kWh

NOx

NOx

g/kWh

Injection start, deg bTDC

Fig. 2 Influence of hydrogen injection start (n = 2000 rpm,  = 1.5, injection

pressure 80 bar, charging pressure 2 bar, ignition 5 deg aTDC)

be

be

be

g/kWh

NOx

NOx

g/kWh

Injection pressure, bar

Fig. 3 Influence of the hydrogen injection pressure (n = 3000 rpm,  = 1.5, injection start 10 deg bTDC, ignition 5 deg aTDC,  charg. pressure           2 bar,           3 bar)

be

be

g/kWh

NOx

NOx

g/kWh

Air excess ratio, 

Fig. 4 Influence of combustion mixture quality (n = 3000 rpm, mep = 10 bar, injection pressure 100 bar, charg. pressure variable, ignition 2 deg aTDC)

be

be

g/kWh

NOx

NOx

g/kWh

deg aTDC

deg bTDC

Fig. 5 Influence of ignition timing (n = 3000 rpm,  = 2.5, mep = 10 bar,

charging pressure 2.5 bar, injection pressure 100 bar)