SOLAR ENERGY DEMONSTRATION ZONES IN THE DALMATIAN REGION

Branimir Hrastnik, Bernard Franković*

Energy Institute “Hrvoje Požar”, Ulica grada Vukovara 37, HR-10000 Zagreb, Croatia,

Phone: +385 1 632 2598, Fax: +385 1 611 8401, e-mail: bhrastni@eihp.hr

*Faculty of Engineering University of Rijeka, Vukovarska 58, HR-51000 Rijeka, Croatia

Phone: +385 51 651 403, Fax: +385 51 675 801, e-mail: bernardf@riteh.hr

ABSTRACT

The energy consumption in the Dalmatian region was estimated for residential and public sector, tourism, commercial sector and industry. The national energy program for the use of solar energy, SUNEN, assessed solar energy potential in Croatia. Energy from fossil fuels and electricity consumption in the region, which is mostly used in households for preparing of hot water and space heating, could be economically substituted by renewable energy. The situation is most promising for islands of Adriatic, where solar thermal collectors, PV modules and wind generators could substitute conventional energy sources in satisfying the present thermal and electric demand. Dalmatian islands, characterised by small density of energy consumption, are proposed as unique candidates in Europe for the nomination as renewable zones, which could demonstrate the full potential of the renewable energy option. As a practical demonstration, the island of Lastovo and the planned tourist village and yacht marina in the Bay of Jurjeva luka are proposed as a first solar demonstration project on the islands. Technical, economic, legal and institutional barriers, as well as shortages of financing the project identification process produced hereto an adverse environment for solar applications in Croatia. This paper is an initiative to eliminate the barriers and intensify the solar energy use in Croatia providing the clean environment and activation of indigenous energy resources in the region.

KEYWORDS

Energy consumption, Dalmatian region, National energy program SUNEN, solar energy, solar demonstration projects.

INTRODUCTION

The geographical position, Adriatic Sea, weather conditions and mild climate make all seven littoral provinces in Croatia best suitable for the use of solar and wind energy. In particular the Adriatic islands have the highest insolation level of 2,300 – 2,800 sunshine hours per year. Islands are the best potential candidates for economic use of solar energy in Croatia (Fig.1.). Croatia has 718 islands with the total area of 3,113 km2, peninsula Pelješac and 389 cliffs and 78 reefs. There are 15 large (area > 30 km2) permanently settled “carrier” islands, 28 middle and smaller (area 1 - 30 km2) “escort”, also populated islands, and 33 smaller (1 - 15 km2) still unpopulated islands.

The appeal of the Adriatic coast and islands

The coastal areas and islands, outside urban areas of larger cities along the coast, are rather sparsely populated. This area with mostly rural characteristics, Mediterranean climate, untouched nature and clean maritime zone is the virgin environment unique in Europe. The region is the best place for holidays, leisure, recreation, sport, medical treatments and many other activities, like culture, arts and art design. It offers potentially excellent opportunities in tourism and supporting services, but also in other branches of economy. The tourist attractiveness of the Adriatic coast and islands originates from the characteristics of the physical environment and the combination of social and cultural image of the Mediterranean region as the “cradle of civilisations”. The combination of these factors together with its geographical accessibility makes it a popular tourist destination. Low tidal amplitudes facilitate the development of seaside resorts and enable also cruise ships to reach any coastal and island location.

Islands are natural ecological resources of healthy agricultural production, of early vegetables (also in counter season), fruits, grapes and olives and of uncontaminated seafood and sea salt. Islands are ideal for bee farming due to abundant habitats of aromatic and medicinal herbs (lavender, rosemary, marjoram, sage, etc.). Also, islands with their meager and healing vegetation are ideal for sheep farming and goat-breeding with a unique taste and quality of their meat. Among all traditional activities well known are both, wine- and olive oil productions.

For more than fifty years, coastal and island destinations look after tourism and supporting services. The present tourist accommodation capacities are 137,500 and 37,900 beds in coastal and island hotels, respectively. There are also motor-camp capacities for 208,000 and 60,900 persons in coastal and island destinations, respectively. In addition, many local inhabitants and residents of larger coastal and continental cities have their spacious weekend houses and comfortable apartments for holidays and leisure. They offer private accommodation in 20,000 and 14,000 apartments along the coast and on islands for tourists, respectively. Many of those capacities offer their rooms for board from May to October. Residential and public buildings, hotels, tourist villages, restaurants and other business sectors need electric energy, as well as energy for heating and cooling. It has been expected that energy consumption could grow by 7.5% per year, which means that energy demand in the region will be doubled by 2010.

The coastal area and islands are quite unsuitable for mass industrial manufacturing, energy production and incineration plants, which could result in adverse liquid and/or gas effluents or solid wastes (Hrastnik, 1999). The aim of this paper is to show the potential, that indigenous energy sources like solar-, wind- and small (wind-pumped) hydro-energy plants could by 100% cover (at least) the expected growth of island’s energy consumption in the 21st century. The solar and wind (primary) energy sources are microeconomically competitive by the local climatic conditions.

Development opportunities of the region

Higher energy and transportation costs for goods and people, time-consuming and mostly quite insufficient daily connections with main land and particularly with other islands, lesser job opportunities, education difficulties, poorer medical care, long distance from urban cultural and art centres and lesser entertainment opportunities make islands at the very start less competitive to the coastal and hinterland regions in the country. During the last fifty years most young people emigrated from the islands. The proper usage of natural and most of renewable resources is an answer how to motivate young people to return and stay on islands. Fast local sea transport, global telecommunication and diverse (also expert) job opportunities, will be the strongest motivation. Also, the islands development policy must change. Policy towards islands must not be created in the regional densely populated coastal cities, which year after year need bigger budgets as to solve their acute urban needs and social problems. On the other side, developing Adriatic islands is not a problem of Croatia alone. In the near future Croatia will be also a part of Europe. The development of the region and thereof long-term business opportunities represent challenges for many existing European interdisciplinary teams of today.

Fig. 1. Geographical position and solar insolation in Croatia in (h/a)

Bilateral and multilateral co-operation opportunities

Most of the islands need high qualified consulting experts in building, solar and wind energy technologies, water-supply, agriculture, manufacturing capacities, computer software, telecommunication, transport logistics, waste-management, fire-protection, and many others. The potential island’s GDP could be higher by a factor of ten, if bilateral and multilateral international cooperation could bring the necessary expert teams together, who are familiar with new technologies and new ideas. Croatia has neither enough experts nor available funds to do it alone in the next 10 - 20 years, which appears as large disadvantage both for Croats and the whole Europe.

Sustainable infrastructure

The islands’ infrastructure, which is bearing much resemblance to the coastal, continental or urban one, can not be made sustainable. The problems of water- and energy supply, means and routes of transportation, goods distribution infrastructure (storehouses, cold storage capacities, fleets of transport boats and vehicles, ships for special purposes, etc.), waste treatment systems and others, must be adopted in a unique way to adapt on islands’ local conditions and categories of geographical, climatic, social, economy, education and other needs. A proper design of infrastructure is the starting point of every technically feasible and economically viable development and implementation plan. To work off this task properly, international teams of experts again will be necessary.

POPULATION

The total population of seven littoral (Adriatic) counties is 1.58 millions, who live in 501,000 households (Fig.2). The islands’ population which counts 126,025 persons lives in 39,643 households. The islands’ households are in average rather old and small (3.2 persons per household). The population density is quite low - on islands live in average 40 persons per square kilometre.

Fig. 2. Geographical and residental data for Adriatic islands

THE ISLANDS’ ENERGY CONSUMPTION

Consumption structure

The characteristics of the island’s energy consumption in households (70% of total consumption) are high fractions of wood and electricity use. The most of electricity in households is used in preparation of hot water (HW) and for space heating (SH). Final energy consumption in an average island household is presented in Table 1.

Table 1. Energy consumption of an average island’s household

| Purpose | Electric energy | Heating oil | LPG | Wood | Totals GJ/a |
| --- | --- | --- | --- | --- | --- |
| Heating | 9.6 | 3.1 | 4.3 | 24.3 | 41.3 |
| -SH | 3.9 | 3.1 | 0.2 | 22.4 | 29.6 |
| -HW | 3.7 |  |  | 0.4 | 4.1 |
| -Cooking | 2.0 |  | 4.1 | 1.5 | 7.6 |
| Cooling | 0.2 |  |  |  | 0.2 |
| Light & Appliances | 8.2 |  |  |  | 8.2 |
| GJ/a | 18.0 | 3.1 | 4.3 | 24.3 | 49.7 |
| Fractions: | 36.2% | 6.2% | 8.7% | 48.9% | 100% |

Energy consumption in sectors like households, industry, tourism, commercial services and agriculture, listed by consumed energy carriers is shown on following figures.

#### Island households

Croatian islands have in total 39,643 households; their final energy consumption is shown in Fig. 3. Nearly 50% of the final energy consumption in the islands’ households of 1970 TJ/a is presently covered by wood. More than one third of energy consumption (mostly for heat purposes) is supplied by electricity (0.2 TWeh).

Fig. 3. Final energy consumption in all island households

#### Islands’ industry

Only about 262 TJ/a is an industrial energy consumption on islands, where 39% is covered by electricity and 61% by heating oil and LPG (Fig. 4.).

Fig. 4. Final energy consumption in islands’ industry (total amount 261,7 TJ/a)

Tourism and catering

The energy consumption in tourism and catering is 322 TJ/a, where most energy is used for space heating and preparing of hot water. Electricity (71%) and heating oil (29%) practically cover the energy demand (Fig. 5.).

Fig. 5. Final energy consumption in island’s tourism (total amount 322.2 TJ/a)

Commercial services and premises

In commercial services (tourism excluded) 79.3% of the energy consumption (61 TJ/a) is covered by electric energy. Heating oil covers the remaining 21% of energy consumption.

Fig. 6. Final energy consumption in commercial services and premises (total amount 61 TJ/a)

#### Island agriculture

The energy consumption in island’s agriculture is small and has been estimated to 21.9 TJ/a. One quarter is covered by electricity and the other three quarters are divided among heating oil, wood and LPG (Fig. 7.).

Fig. 7. Final energy consumption in island’s agriculture (total amount 21.9 TJ/a)

In the summary energy consumption by sectors and energy supply by carriers is shown in Fig. 8. and Fig. 9.

Fig. 8. Final energy use by sectors

Fig. 9. Final energy use by energy carriers

Final energy consumption per capita and unit area

The final energy consumption on islands per capita is in average 20.9 GJ (5.8 MWh) and the final energy consumption per unit area is in average 847.1 GJ/km2 (235.3 MWh/km2).

SOLAR ENERGY POTENTIAL

The assessment of the solar energy potential in Croatia is a result of the national Solar Energy Program SUNEN (Hrastnik et al., 1998). Global solar radiation on south-facing 45° tilted surface for littoral provinces, Split and Lastovo, is shown in Fig. 10.

The total energy available from the solar radiation, which is the natural potential within the seven littoral provinces of area of 24,696 km2 is over 125,600 PJ/a. The technical potential is a fraction of the natural potential, which depends on the physical availability of the land and on the efficiency of the technologies used to collect and convert solar energy into other useful forms of heat, cold, electricity, power or light. The littoral technical potential was assessed well over 1,200 PJ/a. The assessment of the littoral economy potential involves assessments of diverse resources; it is defined as that part of the technically available energy or power harvest, which can be exploited under present or future economic circumstances using existing or modified current technologies. The economy potential of (seven) littoral provinces of 22 PJ/a has been estimated.

Fig. 10. Global solar radiation on south facing surfaces

The exploitable renewable energy potential in littoral provinces consisting of solar energy (well above 120 PJ), wind energy (3.6 PJ) and already installed hydroelectric power of 1,641 MWel substantially prevail the present energy demand in the Adriatic region. These facts are even more valid for Dalmatian islands with high solar (19 PJ) and wind energy potential (1.6 PJ) on one side, and rather low (rural) energy consumption on the other side. Islands like Hvar, Vis, Korčula or Lastovo have more than 2,700 solar hours per year. For the illustration, the field of solar collectors having an area of the football stadium, located on the one of those islands, would produce in energy transformation 45 TJ/a of final thermal energy, of which is 20 TJ/a the useful (end-use) energy. The total final energy demand of littoral provinces with a population of 1.6 millions is roughly 50 PJ, where final energy of 30 PJ is used to cover the low temperature thermal demand. Most of this distributed demand is used for hot water preparation and space heating, which are covered by heating oil and electricity. Over 95% of energy used in preparing HW is electricity. In rural areas most space heating is covered by wood. Using last generation of flat plate selective solar collectors on the mass scale, the distributed energy demand could be supplied from solar collector fields with an area, which corresponds to 800 football stadiums (collector area of 6 million square meters). It literally means that solar collector fields of total area which is only 0.25 per mille of littoral territory, or 2 per mille of the islands’ territory, could by 100% satisfy the low temperature thermal demand in the Adriatic region.

As a conclusion many small hybrid plants for district heating and power supply, using solar, wind and hydro power including reversible wind-pumped power plants, could by 100% substitute the energy demand, which is at present dominantly (60 %) covered by the fossil fuels. As a practical example and illustration, more than 0.5 TWh/a of electric energy, which is presently used for preparing hot water in littoral provinces, could be immediately substituted by the solar energy from flat plate collectors and absorbers. In the same way, at least 1.5 TJ/a of final energy presently used for space heating could be substituted by solar energy.

The potential for PV installations is also enormous in the region. In the region prevail rural areas, particularly on islands. For isolated users and smaller settlements the price for the grid-connection is rather high and money could be better used for investments in distributed (decentralised) off-grid PV-roof systems. For most isolated users, PV-power installations will be needed in the range of 1-20 kWp. The total interest of potential users at the time is estimated to 2.5 MWp. The expectations are that Croatian PV-market could grow in next ten years by 20% per year.

The overall building activity in the coastal regions is a best demonstration of a totally obsolete practice. Most of the presently built houses are based on old habits, sloppy work and traditional out of time materials and technologies. Most of such buildings still need more than 150 kWh/m2a for heating and cooling. The spatial and architectural planning of new residential and tourist settlements as a rule turn no attention to the passive use of solar energy, which together with new materials and technologies could result in energy consumption under 150 MJ/m2a (Miščević, 1993, 1995). There are again many opportunities for well qualified European building companies, which should enter in the region as soon as practically achievable.

A CONCEPT OF RET DEMONSTRATION ZONES

Sustainable development of the Adriatic islands

The fragile and vulnerable nature of island environment means that importing inappropriate energy models also represents a serious risk to island environment, their most valuable asset. Due to its territorial, environmental and economic implications the energy represents a central element of the islands’ dilemma. The adoption of improper models of energy supply could be detrimental for islands’ economy and environment, as energy solution is closely related to a way in which insular resources are managed. This interdependence is extremely evident in islands, also involving transport, water or residues policies that are key aspects, which ought to be satisfactorily resolved.

On the other hand, they generally have an abundance of available RES that make it possible to install distributed, in many cases stand-alone RET systems. The majority of islands posses abundant natural resources, such as wind and sun, which as primary energy resources can be transformed to the useful energies like heat, cold and electricity. Many Adriatic islands could be made self-reliant by use of available solar and wind energy potential, in some cases also by use of wind-pumped hydro accumulation plants.

- A pledge for self-reliance of the islands based on RES

- Most Adriatic islands of the virgin beauty and unique in nature are still ecologically preserved. Small number of residents, very low level of industry production and no mass tourism keep their energy demand density at very low levels. Low energy demand density makes them ideal for renewable energy applications. The risks of fossil fuels endanger the natural balance of the ecological system as well as the traditional activities of the population. Electricity as a clean source of energy is generally compatible with tourism and traditional activities, but electrification of mostly rural areas on islands is never an economic category. Local production of electricity based on fossil fuels is ecologically unacceptable. If solar and wind energy were used to produce power, no adverse impacts on the environment would be present. Due to local climatic and natural conditions, which are extremely favourable for using solar and wind energy, such applications are already competitive and economically viable (Hrastnik, 1999).

- The island of Lastovo - a potential demonstration zone

- The most of Lastovo population lives in two communities - Lastovo (734 residents) and Ubli (303 residents). Other population (168) is distributed among three smaller communities: Pasadru (79), Zaklopatica (69) and Skrivena Luka (20). Their traditional activities are wine- and olive growing as well as fishing. Access to island of Lastovo was forbidden for foreign tourists for fifty years due to military installations. Nowadays, there is no military presence anymore and island is again “a park of nature” developing a new image of a distinguishable tourist destination. Nearby island of Prežba bridged to Lastovo island makes two deep bays “Veliki Lago” and “Mali Lago”, where “Jurjeva luka” (Port of Saint Juraj) is the most interior part of the bay Mali Lago. Potable water from a single desalination plant with a capacity of 4 l/s is rather short for the peak demand of tourists in summer. Also, no enough irrigation water (sources of brackish water of a capacity 430 m3/d contain higher levels of dissolved salts) for Lastovo’s agriculture, which is a complementary activity to the tourism, make it difficult to prosper during the summer. The 27-km long cable connects Lastovo to the national electric grid. However, further expansion of the local net is not economically viable at the present level of (low) electricity consumption. Prospects for using local wind generators and photovoltaic installations are quite promising. Such local installations will become necessary if in five years period 100-200 thousands of tourist-nights are to be realised (Gal and Miščević, 1999).

- Renewable energy supply for the island of Lastovo

- The present final energy demand for island of Lastovo is 22.3 TJ, out of which 80.7% is used in households. The present energy supply consists of electric energy (35%), heating oil (25%) and wood (40%). The final thermal and electric energy consumption as used in households, tourism, other commercial services, public services, handicraft, agriculture and for desalination is presented in Table 3.

- Table 3. Final energy consumption on the island of Lastovo in year 2000

|  | Energy consumption in GJ/a |  |  |  |
| --- | --- | --- | --- | --- |
| Purpose | Electric energy | Heating oil | Wood | Totals |
| Households | 4,900 | 3,850 | 8,749 | 17,499 |
| Tourism | 225 | 899 | 0 | 1,124 |
| Other commercial services | 175 | 408 | 0 | 583 |
| Public services | 445 | 297 | 0 | 742 |
| Handicraft | 103 | 68 | 0 | 171 |
| Agriculture | 79 | 152 | 99 | 330 |
| Desalination | 1,858 | 0 | 0 | 1,858 |
| GJ/a: | 7,784 | 5,674 | 8,848 | 22,306 |
| Fractions: | 34.9% | 25.4% | 39.7% | 100% |

- Predictable energy consumption by year 2010 was estimated assuming the yearly growth of energy consumption in sectors like households (7%), tourism (12%), other commercial services (6%), public services (5%), handicraft (8%), agriculture (5%) and desalination capacity (10%). The energy demand forecast by year 2010 is shown in Table 4.

- Table 4. Forecast of the final energy consumption by 2010

|  | Predicted energy demand in GJ/a |  |  |
| --- | --- | --- | --- |
| - Purpose | - Electric | - Thermal | - Totals |
| Households | 9,638 | 24,784 | 34,422 |
| Tourism | 698 | 2,792 | 3,490 |
| Other commercial services | 313 | 731 | 1,044 |
| Public services | 725 | 484 | 1,209 |
| Handicraft | 222 | 148 | 369 |
| Agriculture | 129 | 408 | 537 |
| Desalination | 4818 | 0 | 4818 |
| GJ/a: | 16,543 | 29,347 | 45,890 |
| Fractions: | 36.1% | 63.9% | 100% |

- The energy planning for the Lastovo island (Vujčić, 1995) is aiming to achieve the 100% self-sufficiency of the island by the year 2010 based on RES.

- One wind generator unit, e.g. Micon NM 750, located at Lastovos macro location Hum would yearly produce 2.56 GWeh (Table 5), which is still in excess when comparing to the present electric energy demand. The presently 0.4 GWeh surplus energy production, which will gradually decrease by future increase of electric energy consumption, shall be fed to the public grid (Majstrović and Horváth, 2000). The estimated 2010/2000 electricity consumption Index (Index 2000 = 100) of 213 shows doubling of electricity consumption in the ten years period. Such growth of electricity consumption could be covered by instalment of an additional wind turbine of 750 kWe.

Table 5. Comparison of Micon 750 kW unit production and present electricity demand for

the island of Lastovo

| - Month | - Production | - Demand | -  grid |
| --- | --- | --- | --- |
|  | - MWh | - MWh | - MWh |
| - Jan | 230 | 264 | -34 |
| - Feb | 237 | 155 | 82 |
| - Mar | 230 | 118 | 112 |
| - Apr | 226 | 107 | 119 |
| - May | 205 | 111 | 94 |
| - Jun | 193 | 182 | 11 |
| - Jul | 182 | 240 | -58 |
| - Aug | 184 | 240 | -56 |
| - Sep | 186 | 207 | -21 |
| - Oct | 205 | 117 | 88 |
| - Nov | 231 | 157 | 74 |
| - Dec | 251 | 264 | -13 |
| - Yearly | 2,560 | 2,162 | 398 |

- Marina and tourist village in the Bay of Jurjeva Luka

The Project of Jurjeva Luka is a part of the more extensive study of the prospects of integral development of Lastovo tourism elaborated by the Consortium for the Development of the Lastovo Archipelago. The results of elaboration was that island of Lastovo has good prospects in tourism, particularly in nautical tourism. From comparison of diverse micro locations in Lastovo archipelago, the conclusion of the Consortium was that the Bay of Jurjeva Luka is best protected against winds and most suitable for giving shelter to boaters. Apart from marina, building of a 4-star apartment settlement of 8000 m2 useful area (200 rooms with 500-600 beds) is being planned. Additional tourist attractions like restaurants, shopping centre, amusement centre, tennis courts, swimming pools, public beach and promenade are planned. The spatial layout and architectural design of apartment village, restaurants, shopping and amusement centres are being planned for best passive use of solar energy. All buildings are planned as low-energy consumption houses with heating demand which should not be exceed 150 MJ/m2a. Active solar roof systems with thermal absorbers and PV modules will make most buildings self-sufficient. The architectural design shall prevent any summer overheating of buildings. Good design of passive ventilation systems will much reduce daily air-conditioning loads in summer. The thermal energy demand for the marina and other tourist attractions in Jurjeva Luka has been estimated to 2.8 TJ/a and estimated electric energy demand is 1.7 GWeh/a. If 100 kWe heat pump would be installed, it could cover the heating and cooling loads at all times. The additional electric consumption for the heat pump would be 177 MWeh/a, which would result in electric consumption of 1.9 GWeh/a covering the total energy consumption of the tourist village and marina Jurjeva Luka. Once marina and village would be fully in function, the electric energy demand of the island of Lastovo will be doubled (3.9 GWeh/a) with the peak electricity demand of 1.5 MWe. The coverage of such demand is possible by two wind generators, 750 kWe each. At the present stage of planning no detailed solutions concerning solar thermal (collector/absorber) and PV capacities are available. An additional advantage of the concept of distributed (local) solar/wind power is that the expected thermal and electric demand growth could be followed by simple extension of solar and wind power and energy supply systems.

- CONCLUSIONS

- Modelling of the energy load and supply diagram for the island of Lastovo have shown the prospects of using renewable energy sources instead of conventional ones. The 100% coverage of energy and water supply demand by renewable sources revealed also the same opportunities for other attractive tourist destinations on other Dalmatian islands Mljet, Korčula, Hvar, Brač, Šolta and some 30-50 other equally attractive islands for the tourism. The best Adriatic holiday resorts deserve the proposed sustainable development approach as an everlasting warranty of meteorological, climatic and ecological balance of the islands’ unique nature. The region is a part of the natural and cultural resource of the Mediterranean and it should also persist as such in the future for many generations to come.

REFERENCES

Hrastnik, B. (1999). The Role of Indigenous Energy Carriers in the Sustainable Development of Croatia. In: Proceedings of Faculty of Economics University of Rijeka – Journal of Economics and Business, Vol.1, pp. 31-53.

Hrastnik, B. et al. (1998). SUNEN – Programme of solar energy use. Energy Institute “Hrvoje Požar”, Zagreb, Croatia.

Miščević, Lj. (1993). Solar Architecture in Croatia. In: Proceedings of ISES Solar World Congress. Budapest Hungary.

Miščević, Lj. (1993). Bioclimatic Rehabilitation of Dwellings in Croatia. Sun at Work in Europe, Vol. 8, 3. The Franklyn Company Consultants Ltd., Birmingham, UK.

Miščević, Lj. (1995). Renewable Energy Technology and Bioclimatic Architecture in Croatia. In: Proceedings of ISES Solar World Congress. Harare, Zimbabwe.

Hrastnik, B. et al. (1998). Prospects and Problems of Solar Energy Development in Croatia. In: Proceedings of Conference on Energy Management, pp.51-54. Dubrovnik, Croatia.

Hrastnik, B. (1999). In: Energy Sector Development Strategy of the Republic of Croatia (G. Granić, ed.), Chap. 2 and 6. Ministry of Economic Affairs of the Republic of Croatia, Zagreb.

Hrastnik B. et al. (1999). The Role of Solar and Wind Energy in Sustainable Development of the Adriatic Macro-Region in Croatia. In: Proceedings of Silver Jubilee Conference “Towards a renewable Future”. ISES UK.

Gal, G. and Lj. Miščević (1999). Port of Saint Juraj – Energy and Ecology Concept. The Consortium for the Complete and Coordinate Development of the Lastovo Archipelago, PROing, SUNARH, Zagreb, Croatia.

Vujčić R. (1995). Solar/Wind Hydrogen Energy System for Adriatic Islands. In: Proceedings of Int. Symposium on Energy, Environment and Economics. Melbourne, Australia.

Franković, B., B. Hrastnik and C. Jedriško (1998). Thermal Utilisation of Solar Energy – Croatian National Energy Program SUNEN, 1998 – 2010. Renewable Energy, 16, 656 – 659.

Hrastnik, B. et al. (2000). Solar and Wind Energy Demonstration Zones in the Dalmatian Region. In: Proceedings of Eurosun 2000. Copenhagen, Denmark.