PROBABILISTIC ANALYSIS OF BOLTED RHS END-PLATE JOINTS IN AXIAL TENSION

Josip Piškovića, Darko Dujmovića,c, Boris Androićb,c,

a  Department of Structural Engineering, Faculty of Civil Engineering, University of Zagreb, Croatia

b IA Projektiranje Structural Engineering Ltd., c Croatian Academy of Engineering, Zagreb, Croatia

jpiskovic@grad.hr, dujmovic@grad.hr, androic@iaprojektiranje.com

Introduction

Bolted end-plate joints have always been a suitable solution for Rectangular Hollow Section (RHS) tension members jointed on building site. In such joints bolting can be provided only along two sides or along all four sides of a hollow section. This paper deals with the joints bolted along two opposite sides (Fig.1). EN 1993-1-8 [1] does not give recommendations for calculation of resistance of such joints. The suitability of EC3 model used for I and H (IHS) section joints with extended end plate to design of RHS joints is questionable. The main difference between bolted IHS and RHS end plate joints is yielding of RHS adjacent to the hogging plastic hinge and participating in the general failure mechanism. Design models from relevant AISC [2] and CIDECT [3] Design Guides can be used with modification of partial factors according to Eurocode. In this paper joints with ‘intermediate plate behaviour’ are investigated. Joint resistances are calculated using models from [1], [2] and [3] and compared with test results from literature [4], [5]. Indicator of model accuracy is mean of test to predicted ratio and coefficient of variation. A better comparison can be achieved by calculating reliability index  of joint by means of probabilistic analysis and comparing with target reliability index target. In reliability analyses it is important to consider the uncertainties in the calculation model as well as the uncertainties in the resistance parameters and applied loads.

,1

,1

,1

,1

RHS

End plate

Bolt

A

A

Section A-A

Weld

bp

e2

e1

tp

t1

d0

p

Fig. 1.  End plate joint with bolts along two sides of RHS

# Resistance OF BOLTED RHS END-PLATE JOINTS IN AXIAL TENSION

## Experimental tension resistance

Kato and Mukai [4] performed two (labeled S1 and S2), and Packer et al. [5] performed series of 16 tests (labeled LB1-LB16) on bolted RHS end-plate joints in axial tension with bolts along only two opposite sides of the RHS. Geometrical and mechanical characteristics of specimens and failure loads from these two experimental studies are used in this paper. Some of the specimens developed end plate mechanism, (‘thin plate behavior’), but all eventually failed by bolt fracture. Namely, end plate mechanism is a theoretical failure mode and does not represent the ultimate joint resistance. The end plate is able to resist additional forces because of strain hardening and membrane effects. The specimens that failed by first failure mode (‘thin end plate configuration’) according to Eurocode (LB2, LB13 LB14) are not considered in this study as well as specimens failed by tube tearing (LB11, LB12, LB16) [5]. For all specimens considered in this study, measured ultimate joint resistances are reported in Table 1.

## EN 1993-1-8 model for I/H section joints

According to Eurocode EN 1993-1-8 [1] three failure modes have to be checked and the lowest is governing. The resistances of all joints considered are governed by second mode, i.e. bolt failure with yielding of end plate given in Eq. (1)

FT,2,Rd=2 Mpl,2,Rd+e2ΣFt,Rde1−lw+e2		(1)

Where	Mpl,2,Rd is design plastic bending resistance of end plate,

Ft,Rd is design tension resistance of bolt,

e1, e2, lw defined in Figure 1 and Table 3.

The characteristic resistances calculated using Eurocode model compared to test results are given in Table 1.

## Resistance according to AISC

In AISC Design Manual [2] a modified T-stub [6] design procedure is used to evaluate the connection limit states. In Struik and de Back’s T stub prying model [7] the term α has been used to represent the ratio of the (sagging) bending moment per unit plate width at the bolt line, to the bending moment per unit plate width at the inner (hogging) plastic hinge. Thus, for the limiting case of a rigid plate, α = 0, and for the limiting case of a flexible plate in double curvature with plastic hinges occurring both at the bolt line and the edge of the T stub web, α = 1.0. Hence, the term α in Struik and de Back’s model was restricted to the range 0  α  1.0. For bolted RHS end-plate joints, this range of validity for α was changed to simply α  0. This implies that the sagging moment per unit width at the bolt line is allowed to exceed the hogging moment per unit width, which was proposed because the RHS member tends to yield adjacent to the hogging plastic hinge and participate in the general failure mechanism. Also, the distance e1 was adjusted to e1’, where: e1’=e1-(d/2)+t1 and e2’=e2+(d/2).

The limit states for the end-plate connection bolted on two sides, are: yielding of the end plate and tensile resistance of the bolts, including prying action Eq.(2). Partial factors for actions and resistance according to Eurocode are adopted. After substitution and rearrangement the expression resistance verification can be written as.

Ft,Ed=FT,Edn1+e1'e2'−Mple2'≤F	(2)

where	FT,Ed is design tension force acting on joint

Ft,Ed is design tension force acting on bolt (including prying)

all other variables are defined in Table 3

The characteristic resistances calculated using AISC model compared to test results are given in Table 1. Since the action force per bolt is calculated, and not the resistance,

## Resistance according to CIDECT

In CIDECT [3] the modified T-stub design procedure is used. Advantage of this model compared to AISC is that design resistance of bolt is used instead of action force. For α = 0 (no prying action), with the bolts loaded to their tensile strength, the required end-plate thickness is "tc". The appropriate value of "α" for a connection was expressed by comparing the end-plate thickness "tp" of a connection with "tc". After substitution and rearrangement the expression can be written as:

FT,Rd=n 0,9Mpl,Rde1'1−e2'e2'+e1'+e2'e2'+e1'Ft,Rd≥FT,Ed						(3)

The characteristic resistances calculated using CIDECT model compared to test results, are given in Table 1.

Table 1.	Comparison of actual to predicted joint resistances

|  | TEST |  | EN 1993-1-8 |  | AISC |  | CIDECT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Joint FT | Bolt Ft | Joint resistance FT,2,Rk | FT/ FT2,Rk | Bolt tension Ft,Ek | Ft /Ft,Ek | Joint resistance FT,Rk | FT/ FT,Rk |
|  | kN | kN | kN |  | kN |  | kN |  |
| LB-1 | 443 | 136 | 413 | 1,07 | 159 | 0,86 | 379 | 1,12 |
| LB-3 | 622 | 221 | 635 | 0,98 | 220 | 1,00 | 583 | 1,07 |
| LB-4 | 793 | 202 | 778 | 1,02 | 212 | 0,95 | 760 | 1,04 |
| LB-5 | 860 | 209 | 897 | 0,96 | 201 | 1,04 | 855 | 1,01 |
| LB-6 | 955 | 210 | 1060 | 0,90 | 193 | 1,09 | 977 | 0,98 |
| LB-7 | 971 | 201 | 1090 | 0,89 | 191 | 1,05 | 1012 | 0,96 |
| LB-8 | 974 | 201 | 1120 | 0,87 | 189 | 1,06 | 1022 | 0,95 |
| LB-9 | 795 | 221 | 833 | 0,95 | 219 | 1,01 | 741 | 1,07 |
| LB-10 | 795 | 138 | 836 | 0,95 | 152 | 0,91 | 725 | 1,10 |
| LB-15 | 680 | 135 | 693 | 0,98 | 147 | 0,92 | 618 | 1,10 |
| S1 | 518 | 164 | 525 | 0,99 | 180 | 0,91 | 472 | 1,10 |
| S2 | 650 | 175 | 665 | 0,98 | 199 | 0,88 | 575 | 1,13 |
|  |  |  | Mean | 0,96 |  | 0,97 |  | 1,06 |
|  |  |  | Stdev. | 0,06 |  | 0,08 |  | 0,07 |

# Probabilistic approach

The tension resistance of bolted joint is considered according to three presented analytical models. Random variables of resistance and action are identified with the appropriate distribution function, mean value and standard deviation. Reliability analysis of joint subjected to permanent and snow load is conducted. The aim of a ‘component’ reliability analysis is to estimate the probability of failure

Pf=g(x)≤0fXxdx		(4)

where X is a random vector which contains all uncertain basic variables; fx(x) represents the joint probability density function of the basic random variables; and g(x) is the limit state function corresponding to the failure mode considered and defined such that the failure event corresponds to g(x)  0. Reliability methods are described in a number of papers. The reliability analysis has been performed using STRUREL 8. The reliability indices obtained from the analysis for different failure modes enabled to estimate the reliability degree and to compare studied design models mutually.

## Limit state functions

Limit state functions are defined for three design models Eq(5), Eq(6) and Eq(7). They can be expressed in general form as follows: gi(x)=ri(x)−e(x), where ri(x) and ei(x) are the vectors of basic variables of resistance and action effect respectively.

gEC3(X)=2 bptp2 fyp4+a ⋅n ⋅0,9⋅d2π ⋅fub4a−lw+bfmR−(FE,g+FE,s)fmE=0	(5)

gCIDECT(X)=n 0,9 ((2 bpn)tp2fy4)γM0b−d2+t11−a+d2a+b+t1+a+d2a+b+t1⋅0,9 πd2fub4 γM2fmR−(FE,g+FE,s)fmE=0 	(6)

gAISC(X)=0,9d2π fub4−fmE(FE,g+FE,s)n1+b−d2+t1a+d2−2bpn tp2fyp4a−d2fmE1	(7)

## Basic variables of actions

In the process of evaluation of the basic variables of action effects, the proportion of the permanent and snow design action effect is 40% and 60 % respectively (FEd,g = 0.4 FEd, FEd,s = 0.6 FEd) . The joint is fully utilized ie. the action effect FEd is equal to the design resistance of joint FRd. The basic variable of permanent action has been adopted with mean value equal to characteristic value and variance of 0,01. The snow load has been adopted according to [9]. The characteristic value of snow load is 98% fractal value, and the mean value is calculated.

Table 2.	Basic variables of actions

|  | EC3 |  | AISC |  | CIDECT |  |
| --- | --- | --- | --- | --- | --- | --- |
| FEd (kN) | 355 |  | 295 |  | 321 |  |
|  | g | s | g | s | g | s |
| FEk (kN) | 105 | 142 | 87,6 | 118,2 | 95,1 | 128,4 |
|  (kN) | 105 | 76,6 | 87,6 | 63,7 | 95,1 | 69,2 |
|  (kN) | 1,05 | 25,278 | 0,876 | 21,02 | 0,951 | 22,836 |
| Distribution | Permanent (Feg)–Normal; Snow(Fes) -Gumbel |  |  |  |  |  |

## Basic variables of resistance

Mean values, standard deviations and the corresponding distribution functions of these basic variables are provided in Table 3.

Table 3.	Basic variables of resistance

|  | ID | Distribution | Mean | Stdev |
| --- | --- | --- | --- | --- |
| Plate width | bp | Normal (Gauss) | 150 | 0,18 |
| Plate thickness | tp | Normal (Gauss) | 20,2 | 0,1 |
| RHS wall to bolt center | e1 | Normal (Gauss) | 44,5 | 0,28 |
| Plate edge to bolt center | e2 | Normal (Gauss) | 41,5 | 0,26 |
| Bolt hole | d0 | Normal (Gauss) | 17,5 | 0,2 |
| Plate yield strength | fyp | Lognormal | 300 | 21 |
| Bolt ultimate strength | fub | Lognormal | 750 | 38 |
| Bolt diameter | d | Normal (Gauss) | 15,9 | 0,06 |
| Weld leg size | lw | Normal (Gauss) | 10 | 1 |
| Number of bolts | n | Constant | 4 | - |

## Model uncertainty

Since in developing resistance and action models certain influences are neglected, deviations between analytical and test results are to be expected. This fact is considered by introducing a model uncertainty in resistance fmr and actions fme. The parameters of fmr are mean and standard deviation of the ratio of test to predicted resistance for each analytical model. As for the uncertainty in actions, fme, mean values are adopted as 1 and variance of 0,05. In AISC analytical model, model uncertainty in action effects is introduced with the mean and standard deviation of the ratio of test to predicted bolt load with  =0,97 and  = 0,08.

Table 4.	Basic variables of model uncertainty

|  | EC3 | AISC |  | CIDECT |
| --- | --- | --- | --- | --- |
| Resistance fmr (Normal) | 0,96 | - |  | 1,06 |
|  | 0,06 | - |  | 0,07 |
| Action fme (Lognormal) | 1,00 | 0,97 | 1,00 | 1,00 |
|  | 0,05 | 0,08 | 0,05 | 0,05 |

# Results and discussion of probabilistic analysis

It can be seen that correlation between actual joint resistance and predicted joint resistance is very good for all models, with ratio having mean =0,96 and standard deviation  =0,06 for EC3, =0,97 and  = 0,08 for AISC and  =1,06 and  = 0,07 for CIDECT. For EC3 model the error is slightly on the un-conservative side. For AISC and CIDECT model the error is on conservative side.

Probabilistic analysis has been conducted by STRUREL [8] according to FORM (First Order Reliability Analysis) for three design models. The design working life of the structure was assumed to be 50 years. Thus, according to [10], the target value of reliability index for ULS is adopted, βtarget = 3.8. The reliability index values obtained according to FORM for ULS and different analytical models are given in Table 5. Reliability indices for all failure models are higher than target value of the reliability index target = 3.8.

Table 5.	Reliability indices 

| Analytical model | EC3 | AISC | CIDECT |
| --- | --- | --- | --- |
| Reliability index  | 3.951 | 4.743 | 4,120 |

FORM also provides a sensitivity factors. Generally written as a percentage, these factors allow the ranking of the basic random variables according to their importance in the reliability analysis. The sensitivity factor of the basic variable FE,s has the greatest influence on the reliability index. An extremely high influence of FE,s on the reliability index is anticipated considering the stochastic character of this basic variable.  Among basic resistance variables, the basic variable fyp and fub have the highest influence on the reliability index.

i

Fig. 2.  Sensitivity factors, i , of basic variables

# Conclusion

Using probabilistic analysis taking into account the variability of basic variables of resistance and actions, the reliability of bolted RHS end-plate joints in axial tension designed according 3 analytical models has been investigated. The systematic development of mechanical joint models by a probabilistic approach to the evaluation of the experimental results may improve the existing models, and thus reliability level of component method may come closer to the required one. Similarly, it is necessary to evaluate the reliability of a joint by considering it as a combined series-parallel system. In doing this, the significance of dominating basic variables expressed by sensitivity factor, i , and the sensitivity of the reliability index with regard to their mean value and standard deviation should be considered. In addition to considering the reliability of a joint as a system, the reliability of a joint from the aspect of compliance between the structural behaviors of joint and structural system should also be considered.

References

[1]	European Committee for Standardization (CEN): Eurocode 3: design of steel structures, part 1-8: design of joints, Brussels: CEN; 2005.

[2]	American Institute of Steel Construction AISC, “Hollow structural sections connections manual”, Chicago; 2010.

[3]	Packer J A, Wardenier J, Zhao XL,  van der Vegte GJ, Kurobane Y, Design guide for rectangular hollow section (RHS)  joints under predominantly static loading,  CIDECT (ed.) and Verlag TUV, Rheinland GmbH, Koln, Germany, 2009.

[4]	Kato B, Mukai A, Bolted tension flanges joining square hollow section members– supplement Bolted at two sides of flange,1985.

[5]	Packer JA, Bruno L, Birkemoe PC, “Limit analysis of bolted RHS flange plate joints”, J Struct Eng, 115(9):2226–42,1989;

[6]	Birkemoe PC, Packer JA. Ultimate strength design of bolted tubular tension connections, Proc., Steel Structures—RecentResearch Advances and their Application to Design- Elsevier Applied Science, 153–168, Budva, Yugoslavia; 1986;

[7]	Struik JHA, de Back J, Tests on Bolted T-stubs with respect to a Bolted Beam-to-Column Connection, Stevin Laboratory Report 6-69-13. Delft, The Netherlands: DelftUniv. Of Technology, 1969.

[8]	STRUREL Manual, RCP GmbH, München, 1992.

[9]	Zaninović K, Gajić-Čapka M, Androić B, Džeba I, Dujmović D. Determining the characteristic snow load. Građevinar 2001;53(6):363–78. [in Croatian].

[10] 	European Committee for Standardization (CEN), EN 1990 – Eurocode: Basis of structural design. Brussels; 2002.