CALCULATION OF GLOBAL IRRADIATION OF TILTED SURFACE AND THE ENERGY CONTRIBUTION OF PHOTOVOLTAIC SYSTEM FOR THE REGION OF HVAR

Jadranka Vuletin, Ivan Zulim i Mirjana Grbac

Faculty of Electrical Engineering, Mechanical Engineering and Naval Architecture - Split

Nenad Sikimić

Split - Shipping Company, Split

Key words:

Solar enrgy

Photovoltaic system

Energy contribution

Ključne riječi:

Sunčevo zračenje

Fotonaponski sustav

Energetski doprinos

Monthly average of daily global irradiation on south oriented tilted surface is calculated for the region of Hvar. The long term data of solar insolation measurements as well as the calculations of the irradiation on a horizontal surface, are used. In estimation of the diffuse component of solar radiation the modified relations suitable to our climate region are applied. The colector tilt depedence of various climatic periods has been examined and the energetic contribution of photovoltaic system is estimated.

1. INTRODUCTION

To calculate and design the devices for solar energy conversion it is necessary to know the Solar energy incident on the solar collector surface. Solar irradiation differs from location to location, depends upon the regional climatic conditions and varies during a day, month and year. The meteorological stations in Croatia measure mostly the Solar irradiation incident on a horizontal surface, and in the stations where these measurementsare not made, the irradiation of the horizontal surface is calculated from the data referred to the sunshine duration (insolation), cloudliness etc. Solar irradiation on a tilted surface has not been measured systematically, therefore it is necessary to calculate it for designing solar energy conversion devices.

2. DURATION OF SUNSHINE IN HVAR

The heliograph was set up in Hvar in 1931, and since then the duration of sunshine has been observed, and from those data the solar irradiation on a horizontal collector surface has been estimated.

Monthly average daily global irradiation Hon a horizontal surface is calculated from the average daily value S of relative sunshine duration and average daily value of extraterrestrial irradiation on a horizontal surface Ho for each month, using the Angström-Page relation [1,2]:

H=Ho ( a + bS)

in which the coefficients a and b depend upon the location being observed. The values for a and b calculated for the locations where the solar irradiation on a horizontal surface has been measured could also be used for the locations where those measurements are not made, but have similar climatic conditions. Relative sunshine duration S is defined as the ratio of real insolation s and possible duration of a sunny day (time from sunrise to sunset) Z.

In table 2.1. are given average monthly values of real sunshine duration (), possible duration (), and relative duration () for the period from 1961. to 1980. for Hvar [3] as well as the calculated average daily values of extraterrestrial and global irradiation on a horizontal surface Hfor each month. The values of the coefficients a and b have been estimated for Hvar from calculations of the monthly averages of that coefficients for the locations in which  global irradiation on a horizontal surfaces is measured.

Table 2.1. 	Average monthly values of the real sunshine duration, possible duration, relative duration, and average daily values of extraterrestrial and global irradiation on a horizontal surface

month      (hour)              (hour)            (%)         	Ho(kWh/m2)	 H(kWh/m2)

I	121,1		288		42,0		  3,70			1,72

II	145,5		291		50,0		  5,13			2,68

III	187,0		360		51,9		  7,21			3,93

IV	226,1		390		58,0		  9,37			5,20

V	282,9		431		65,6		10,93 			6,27

VI	317,8		430		73,9		11,59			7,24

VII	362,0		434		83,4		11,24			7,23

VIII	326,5		418		78,1		  9,93			6,19

IX	257,8		366		70,4		  7,93			4,73

X	212,2		335		63,3		  5,73			3,42

XI	123,6		288		42,9		  4,01			1,94

XII	111,7		262		42,6		  3,28			1,45

Besides  the sunshine duration, the number of days without sunshine as well as the number of days with the continuous sunshine can complete sunshine characteristics of a particular location. . The average number of days with continuous sunshine for Hvar in the period 1961-1980, is given in the table 2.2., and the average number of days without sunshine in table 2.3.   [3].

Table 2.2.  The average number of days with continuous sunshine for Hvar:

| month | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| number of days | 4,5 | 6,3 | 4,9 | 6,7 | 6,9 | 9,3 | 14,3 | 13,6 | 12,2 | 7,9 | 4,6 | 2,9 |

Tablica 2.3. The average number of days without sunshine for Hvar:

| month | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| number of days | 6,0 | 3,5 | 3,0 | 1,6 | 0,6 | 0,1 | 0,1 | 0,2 | 0,7 | 1,7 | 4,2 | 6,8 |

3. CALCULATION OF MONTHLY AVERAGE VALUES OF GLOBAL IRRADIATION ON  A TILTED SURFACE

In this paper, the monthly average global irradiation of tilted collector surface Hβ has been calculated using daily global irradiation average value H according to the Liu-Jordan relation [5]:

Hβ =  R×H,

in which R is the monthly average value of tilt factor being given by the expression:

R=1−HdHRb+HdH1+cosβ2+ρ1−cosβ2 ,

where is:

b    -  	collector tilt angle

Hd -  	monthly average daily diffuse component of radiation on a horizontal surface

r    -  	reflectivity factor of ground

Rb  -  	monthly average daily value of the tilt factor referred to the direct component of 	radiation

Monthly average daily diffuse irradiation Hd is calculated from the Liu-Jordan empirical relation [6], being modified for our climatic region [7] is

Hd = (1,60 - 4,17 KT + 5,29KT2 - 2,86KT3) H,

where KT is monthly average value of the clearness index being defined as the ratio of monthly average daily global irradiation of a horizontal surface on the ground H and monthly average daily extraterrestrial global radiation on a horizontral surface Ho:

KT =  H/Ho .

Monthly average daily value of the direct radiation tilt factor Rb can be expressed as the ratio of the monthly average irradiation values for tilted and that on horizontal surface [5]. For the south oriented surfaces, Rb  is given by:

Rb=cos(j−β)cosdsinw'+(p180)w'sin(j−β)sindcosjcosdsinw+(p180)wsinjsind ,

where is:	j - latitude

d -  Sun declination

w - Sun rise angle for a horizontal surface

w = arc cos(- tgj tgd)

w' - Sun rise angle for a tilted surface

w' = min{w, arc cos[-tg(j-b) tgd] }

The calculations are made for average daily values of the clearness index KT, diffuse irradiation of a horizontal surface Hd, and global irradiation of a tilted surface Hβ for Hvar (j=43°10¢)  for the tilt angles: b1 = j ,  b2 = j +15°,  b3 =j -15° . The value of reflectivity factor is taken 0,2. The average values of global irradiation on a horizontal surface H for period 1961. to 1980. (table 2.1.) are used in calculations, and the obtained results are given in tables 3.1. and 3.2.

The average values of daily global irradiation on a tilted surface, Hβ are shown in fig. 3.1.

Table 3.1. The average daily values of the clearness index and corresponding diffuse 	irradiation on a horizontal surface

| month | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KT | 0,465 | 0,522 | 0,545 | 0,555 | 0,573 | 0,625 | 0,643 | 0,623 | 0,596 | 0,596 | 0,483 | 0,443 |
| Hd(kWh/m2) | 0,890 | 1,346 | 1,711 | 0,061 | 0,109 | 1,445 | 2,499 | 2,399 | 1,697 | 4,950 | 3,200 | 2,560 |

Table 3.2. Mean daily global irradiation on a tilted surface of the south oriented collector  Hβ(kWh/m2), for tilt angles j ,j +15o,j -15o, in Hvar:

| month | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII | yearly |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| j +15o | 3,07 | 4,06 | 4,65 | 4,87 | 4,98 | 5,30 | 5,48 | 5,44 | 5,19 | 4,99 | 3,33 | 2,70 | 55,06 |
| j | 2,93 | 3,99 | 4,81 | 5,31 | 5,66 | 6,17 | 6,32 | 6,05 | 5,47 | 4,95 | 3,20 | 2,56 | 57,42 |
| j -15o | 2,63 | 3,71 | 4,72 | 5,51 | 6,11 | 6,80 | 6,91 | 6,38 | 5,47 | 4,64 | 2,90 | 2,28 | 58,06 |

Hβ(kWh/m2)

month

Figure 3.1.  Monthly average values of daily global irradiation on the tilted surface of the 		south oriented collector, for the tilt angles: b1 = j ,  b2 = j +15°,  b3 =j -15° .

The average value of the monthly daily global irradiation on tilted surface is high for Hvar (table 3.1.) throughout a year . For the winter months the optimal angle is j +15°.

4. CALCULATION OF ENERGY CONTRIBUTION FOR A 	PHOTOVOLTAIC SYSTEM

In fig. 4.1. is shown a typical photovoltaic system consisting of a module, load and battery. Battery supplies load with energy during the time when there is no sunshine or in the conditions when the level of produced energy is insufficient to the needs of load. A photovoltaic system can function even without battery during the sunshine, but in that case the produced electric energy has to be immediately used while the excess energy will be irreversibily wasted.

Figure 4.1. Typical configuration of a photovoltaic system

Mean daily electric energy available to the load Ep  equals

Ep = hE                                                   (4.1)

where E is mean daily incident Solar irradiation, and h is the average module efficiency, or the efficency of module and control subsystem.

The incident Solar irradiation is calculated from the average daily global irradiation data for the tilted surface Hβ, so the relation (4.1.) can be written as

Ep = hAHβt  ,                                              (4.2.)

where A is the effective area of the panel and t is the average transmissivity of the panel cover.

For the calculations of the average daily amount of electric energy available to the load, Ep, for several typical module arrangements (the number of modules nm = 4,8,40 i 80) are used the data referred to the irradiation on a tilted surface, Hβ (table 3.2.). The calculation is made for Hvar region using the module type KT1-3A (production of  Končar-Solar Cells Splitu) with effective area A = 0,285m2 , average cover transmissivity t=1, module efficency hm=0,06 and control subsystem efficiency hc = 0,9, for the following values of tilt angles b1=j=43°10¢ (Hvar), b2 = j +15°,  b3 =j -15° . In tables 4.1.a.,b.,c. are given the calculation results for mean daily values of electrical energy Ep, and in tables 4.2.a.,b.,c. mean monthly values of electrical energy available to the load Wp  for several typical system configurations (nm = 4,8,40 i 80 modules) .

Table 4.1.	Average daily values (over the months) of electrical energy available to the 			load for several typical system configurations (nm = 4,8,40 i 80 modules)  			and tilt angles:

a) b =j =43°10¢

|  |  | Ep [kWh] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| month | Hβ [kWh/m2] | nm=4 | nm=8 | nm=40 | nm=80 |
| I | 2,93 | 0,18 | 0,36 | 1,80 | 3,60 |
| II | 3,99 | 0,24 | 0,48 | 2,40 | 4,80 |
| III | 4,81 | 0,30 | 0,60 | 3,00 | 6,00 |
| IV | 5,31 | 0,32 | 0,64 | 3,20 | 6,40 |
| V | 5,66 | 0,35 | 0,70 | 3,50 | 7,00 |
| VI | 6,17 | 0,38 | 0,76 | 3,80 | 7,60 |
| VII | 6,32 | 0,39 | 0,48 | 3,90 | 7,80 |
| VIII | 6,05 | 0,37 | 0,74 | 3,70 | 7,40 |
| IX | 5,47 | 0,33 | 0,66 | 3,30 | 6,60 |
| X | 4,95 | 0,31 | 0,62 | 3,10 | 6,20 |
| XI | 3,20 | 0,20 | 0,40 | 2,00 | 4,00 |
| XII | 2,56 | 0,16 | 0,32 | 1,60 | 3,20 |

b) b =j +15°=58°10¢

|  |  |  | Ep [kWh] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| month | Hβ [kWh/m2] | nm=4 |  | nm=8 | nm=40 | nm=80 |
| I | 3,07 | 0,19 |  | 0,38 | 1,90 | 3,80 |
| II | 4,06 | 0,25 |  | 0,50 | 2,50 | 5,00 |
| III | 4,65 | 0,29 |  | 0,58 | 2,90 | 5,80 |
| IV | 4,87 | 0,30 |  | 0,60 | 3,00 | 6,00 |
| V | 4,98 | 0,31 |  | 0,62 | 3,10 | 6,20 |
| VI | 5,30 | 0,33 |  | 0,66 | 3,30 | 6,60 |
| VII | 5,48 | 0,34 |  | 0,68 | 3,40 | 6,80 |
| VIII | 5,44 | 0,33 |  | 0,66 | 3,30 | 6,60 |
| IX | 5,19 | 0,32 |  | 0,64 | 3,20 | 6,40 |
| X | 4,99 | 0,31 |  | 0,62 | 3,10 | 6,20 |
| XI | 3,33 | 0,20 |  | 0,40 | 2,00 | 4,00 |
| XII | 2,70 | 0,17 |  | 0,34 | 1,70 | 3,40 |

c) b =j -15°=28°10¢

|  |  |  | Ep [kWh] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| month | Hβ [kWh/m2] | nm=4 |  | nm=8 | nm=40 | nm=80 |
| I | 2,63 | 0,16 |  | 0,32 | 1,60 | 3,20 |
| II | 3,71 | 0,23 |  | 0,46 | 2,30 | 4,60 |
| III | 4,72 | 0,29 |  | 0,58 | 2,90 | 5,80 |
| IV | 5,51 | 0,34 |  | 0,68 | 3,40 | 6,80 |
| V | 6,11 | 0,38 |  | 0,76 | 3,80 | 7,60 |
| VI | 6,80 | 0,42 |  | 0,84 | 4,20 | 8,40 |
| VII | 6,91 | 0,42 |  | 0,84 | 4,20 | 8,40 |
| VIII | 6,38 | 0,39 |  | 0,78 | 3,90 | 7,80 |
| IX | 5,47 | 0,34 |  | 0,68 | 3,40 | 6,80 |
| X | 4,64 | 0,28 |  | 0,56 | 2,80 | 5,60 |
| XI | 2,90 | 0,18 |  | 0,36 | 1,80 | 3,60 |
| XII | 2,28 | 0,14 |  | 0,28 | 1,40 | 2,80 |

Table 4.2. 	The average monthly values of electrical energy available to a load for 			several typical system configurations (nm = 4,8,40 i 80 modules), and the 			values of the tilt angles:

a) b =j =43°10¢

|  |  | Wp [kWh] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| month | NHβ [kWh/m2] | nm=4 | nm=8 | nm=40 | nm=80 |
| I | 90,83 | 5,59 | 11,18 | 55,90 | 111,80 |
| II | 111,72 | 6,88 | 13,76 | 68,80 | 137,60 |
| III | 149,11 | 9,18 | 18,36 | 91,8 | 183,60 |
| IV | 159,30 | 9,81 | 19,62 | 98,1 | 196,20 |
| V | 175,46 | 10,80 | 21,60 | 108,00 | 216,00 |
| VI | 185,10 | 11,39 | 22,78 | 113,90 | 227,80 |
| VII | 195,92 | 12,06 | 24,12 | 120,60 | 241,20 |
| VIII | 187,55 | 11,54 | 23,08 | 115,40 | 230,80 |
| IX | 164,10 | 10,10 | 20,20 | 101,00 | 202,00 |
| X | 153,45 | 9,45 | 18,90 | 94,50 | 189,00 |
| XI | 96,00 | 5,91 | 11,82 | 59,10 | 118,20 |
| XII | 79,36 | 4,88 | 9,76 | 48,80 | 97,60 |
| per year | 1747,90 | 107,59 | 215,19 | 1075,90 | 2151,80 |

b) b =j+15° =58°10¢

|  |  | Wp [kWh] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| month | NHβ [kWh/m2] | nm=4 | nm=8 | nm=40 | nm=80 |
| I | 95,17 | 5,86 | 11,72 | 58,60 | 117,20 |
| II | 113,68 | 7,00 | 14,00 | 70,00 | 140,00 |
| III | 144,15 | 8,87 | 17,74 | 88,70 | 177,40 |
| IV | 146,10 | 8,99 | 17,98 | 89,90 | 179,80 |
| V | 154,38 | 9,50 | 19,00 | 95,00 | 190,00 |
| VI | 159,00 | 9,79 | 19,58 | 97,90 | 195,80 |
| VII | 169,88 | 10,46 | 20,92 | 104,60 | 209,20 |
| VIII | 168,64 | 10,38 | 20,76 | 103,80 | 207,60 |
| IX | 155,70 | 9,58 | 19,16 | 95,80 | 191,60 |
| X | 154,69 | 9,52 | 19,04 | 95,20 | 190,40 |
| XI | 99,90 | 6,15 | 12,30 | 61,50 | 123,00 |
| XII | 83,70 | 5,15 | 10,30 | 51,50 | 103,00 |
| per year | 1644,99 | 101,25 | 202,50 | 1012,50 | 2025,00 |

c) b =j-15° =28°10¢

|  |  | Wp [kWh] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| month | NHβ [kWh/m2] | nm=4 | nm=8 | nm=40 | nm=80 |
| I | 81,53 | 5,00 | 10,00 | 50,00 | 100,00 |
| II | 103,88 | 6,39 | 12,78 | 63,90 | 127,80 |
| III | 146,32 | 9,01 | 18,02 | 90,10 | 180,20 |
| IV | 165,30 | 10,18 | 20,36 | 101,80 | 203,60 |
| V | 189,41 | 11,66 | 23,32 | 116,60 | 233,20 |
| VI | 204,00 | 12,56 | 25,12 | 125,60 | 251,20 |
| VII | 214,21 | 13,19 | 26,38 | 131,90 | 263,80 |
| VIII | 197,78 | 12,18 | 24,36 | 121,80 | 243,60 |
| IX | 164,10 | 10,10 | 20,20 | 101,00 | 202,00 |
| X | 143,84 | 8,85 | 17,70 | 88,50 | 177,00 |
| XI | 87,00 | 5,36 | 10,72 | 53,60 | 107,20 |
| XII | 70,68 | 4,35 | 8,70 | 43,50 | 87,00 |
| per year | 1768,05 | 108,83 | 217,66 | 1088,30 | 2176,60 |

5. CONCLUSION

The calculation of global irradiation on a tilted collector surface as well as investigation of the dependence of the irradiation upon the tilt angle during a year make it possible to choose the optimal tilt angle for the winter months, for the summer months, and for the annual average of the acumulated energy.

LITERATURE:

[1]	A. Angström: Computation of Global Radiation from the Records of Sunshine Hours, Arkiv Geophysik 3(1956)556.

[2]	J.K.Page: The Estimation of  Monthly Mean Values of Daily Total Short-Wave Radiation on Vertical and Inclined Surfaces from Sunshine Records for Latitudes 40oN-40 oS, Proceedings of the UN Conference on New Sources of Energy 4(1964)378

[3]	Meteorološki parametri potrebni za iskorištavanje Sunčeve energije u SR Hrvatskoj, I dio - Trajanje sijanja Sunca, Republički hidrometereološki zavod, Centar za metereološka istraživanja, Zagreb, 1981.

[4]	Meteorološki parametri potrebni za iskorištavanje Sunčeve energije u SR Hrvatskoj, III dio - Globalno zračenje, Republički hidrometereološki zavod, Centar za metereološka istraživanja, Zagreb, 1986.

[5]	B.Y.H.Liu and R.C.Jordan: Availability of Solar Energy for Flat-Plate Solar Heat Collectors Daily Insolation on Surface Tilted Toward the Equator, ASHRAE Trans. (1962)526

[6]	B.Y.H.Liu and R.C.Jordan: Availability of Solar Energy for Flat-Plate Solar Heat Collectors, in Applications of Solar Energy for Heating and Cooling of Buildings, ed.R.C.Jordan and B.Y.H.Liu, American Society of Heating, Refrigerating and Air-Conditioning Engineers, Inc., Atlanta, Ga.,1977.

[7]	P.Kulišić:Korelacije između difuznog i globalnog zračenja, Sunčeva energija Vol.6, 	No.1 (1985)7

PRORAČUN GLOBALNE OZRAČENOSTI NAGNUTE PLOHE I ENERGETSKOG DOPRINOSA FOTONAPONSKOG SUSTAVA U PODRUČJU HVARA

Proračunat je mjesečni prosjek dnevne globalne ozračenosti nagnute plohe kolektora okrenutog prema jugu u području Hvara. Korišteni su podaci dugogodišnjih mjerenja insolacije te proračuni ozračenosti horizontalne plohe. Za procjenu difuzne komponente zračenja primjenjivane su modificirane relacije prilagođene našem klimatskom području. Ispitivana je ovisnost o kutu nagiba kolektora u različitim vremenskim razdobljima te je procijenjen energetski doprinos fotonaponskog sustava.